Communication : CAN :

Le Protocol utilisé pour le paramétrage et la mise au point est le CANopen.

Le Protocol utilisé pour la commande du moteur sera le H-protocol, déclinaison du J1939.

Afin d’exploiter le variateur HVLP SEVCON met à disposition un certain nombre de documents dont la liste suit :

- App Note - H-Protocol Applied To Genus:

Ce document décrit la messagerie en J1939 utilisée par le variateur pour piloter le moteur et la procédure à suivre pour le control de la machine d’état animant le variateur.

Voir en particulier

- App Note - Mapping J1939 Items to VehApp:

Ce document décrit la messagerie en J1939 utilisée par le variateur pour piloter le moteur à l’aide des ressources d’Entrées/Sorties.

- Object Dictionary generated for SN0088.37 [SN0089.37]:

Dictionnaire d’Objets utilisés par le protocol CANopen.

- App Note - I-Protocol Applied To Genus:

Ce document décrit le protocol pour l’utilisation des E/S physiques pour le pilotage du moteur.

- *.dbc:

Base de données J1939 pour VECTOR Canalyser.

Mise en réseau :

In addition to in-service operation, the CANopen protocol allows the controller to be commissioned using the Sevcon DVT tool.

Protocol used for motoring operation:  Sevcon’s H-protocol (providing J1939 compatibility) for communications.

HVLP dedicated technical notes:

- App Note - H-Protocol Applied To Genus:

The purpose of this application note is to explain in simple terms what the H-Protocol is, how to use it, its form and configuration.

The H-Protocol provides a simple, fast and flexible way to control Sevcon motor controller ECU’s. It is a J1939 based Peer to Peer message protocol to sequence and control the operation of Sevcon ECU’s. ECU Status message’s are sent back to the controlling unit as J1939 broadcast messages.

- See section 1.3.2 H-Protocol Control Message Types and section 1.3.3 H-Protocol Status Message Types for the messages to manage

- You will have to implement the state machine described in chapter 2 H-Protocol State Machine Control.

- App Note - Mapping J1939 Items to VehApp:

When No VCU is available, the HVLP (Genus) controller offers abilities of a routing system to map H-protocol and I-protocol signals between devices. Both of these protocols use J1939 style messaging formats.

- App Note - I-Protocol Applied To Genus:

Ce document décrit le protocol pour l’utilisation des E/S physiques pour le pilotage du moteur.

protocol for I/O configuration is provided

- Object Dictionary generated for SN0088B45 [SN0089.45]

- The Object Dictionary provides important information about CANopen communication with HVLP-10/20.

- *.dbc:

J1939 Database for VECTOR Canalyser.

- App Note - Genus Line Contactor and Precharge Configuration.pdf

- 20150129.NT.01 SEVCON - Gen4 - HVLP - DC bus capacitance (values) charging time