Procedure :

  - Update controller software to the latest :  mcp_SN0088.37B iop_SN0089.37

To update the controller software   proceed as follow

| tool | Commande/action | commentaires |
| --- | --- | --- |
| DVTC : command | lg 1 | Log into the controller |
| DVTC : command | fpo 1 pre | Request pre-Operationnal mode |
| DVTC : command | bts 1 | Enter in bootloader Mode |
| DVTC : command | load_dld 1 "[tk_getOpenFile]" | Open file : 2203_mcp_ac_application - SN0088.37B.dldx |
| DVTC : command | load_dld 16 "[tk_getOpenFile]" if failed with timeout issue, try : load_dld 17 "[tk_getOpenFile]" | Open file : 2103_lincoln_iop - SN0089.37_a.dldx If failed, try : 2103_lincoln_iop - SN0089.37_9.dldx |
| DVTC : command | bte 1 | Exit from bootloader Mode |
| DVTC : command | fpo 1 op | Request Operationnal mode |
| DVT_helper | click : | Cheking the firmware version (Vérification de la version du programme du variateur) |